IoT and AI-Integrated

Embedded Systems

Build the future of connected intelligent devices.

Workshop Program Document  |  2026

# Table of Contents

Table of Contents	1

Program Overview	1

Prerequisites	1

Software to Install Before Day 1	1

Hardware Kit (Per Participant or Team)	1

Day 1 - Embedded Foundations: Talking to Hardware	1

Topics	1

Hands-on Lab	1

Learning Outcomes	1

Day 2 - Concurrency, Power and Connectivity	1

Topics	1

Hands-on Lab	1

Learning Outcomes	1

Day 3 - Capstone 1: Hospital Helpdesk Voice Chatbot	1

Topics	1

Hands-on Lab / Capstone Build	1

Learning Outcomes	1

Day 4 - Capstone 2: Library AI Voice Assistant	1

Topics	1

Hands-on Lab / Capstone Build	1

Learning Outcomes	1

Day 5 - Capstone 3: AI-Assisted Home Power Management System	1

Topics	1

Hands-on Lab / Capstone Build	1

Learning Outcomes	1

Final Deliverables	1

Demo Format	1

# Program Overview

This workshop takes participants from embedded fundamentals to fully working AI-integrated IoT systems over five days. The progression is deliberate: Days 1-2 build the hardware and firmware foundation every later project depends on, while Days 3-5 each deliver one complete, demoable capstone system combining voice/text AI, databases, and real-world automation.

Each capstone targets a distinct real-world domain - hospital operations, library management, and home energy management - so participants leave with three transferable system designs, not just one.

# Prerequisites

- Basic understanding of microcontroller programming (C/C++ fundamentals, comfort with the Arduino IDE)

- A personal laptop (Windows/Mac/Linux) with administrator rights to install software

- No prior IoT, AI, or voice-assistant experience required

# Software to Install Before Day 1

- Arduino IDE, with ESP32 board support package installed

- Node.js (LTS version) - required to run n8n

- n8n (run locally via npx n8n - no paid account needed)

- A free API key from a generative AI provider (e.g. Google Gemini) - exact provider may vary based on free-tier availability at workshop time; core workflow concepts are provider-agnostic

- A free API key from a weather data provider (e.g. WeatherAPI.com) - used on Day 5

- A Meta Developer account, with a WhatsApp test app created in advance - used on Day 5

Software signups are done before Day 1 so no lab time is lost to account creation or email verification delays.

# Hardware Kit (Per Participant or Team)

| Category | Components |
| --- | --- |
| Core | ESP32 Dev Board, USB data cable, breadboard, jumper wires |
| Sensors | DHT11, BME280, IR or Ultrasonic distance sensor, Soil moisture sensor |
| Audio | INMP441 (I2S digital microphone), MAX98357 (I2S amplifier + speaker) |
| Display and Input | OLED display (I2C, 0.96"), push buttons (x2-3) |
| Location | GPS module |
| Actuation | LEDs (assorted), resistors, relay module (for pump/load simulation) |

BME280 is included alongside DHT11 to give participants exposure to a second I2C sensor with better accuracy (temperature, humidity, and barometric pressure) - useful for the Day 5 environmental logic.

# Day 1 - Embedded Foundations: Talking to Hardware

## Topics

- ESP32 architecture overview - dual-core, GPIO, ADC/DAC, peripheral map

- I2C protocol - addressing, master/slave, wiring the OLED display and BME280

- SPI protocol - wiring and use cases vs. I2C

- UART - serial communication, debugging via Serial Monitor

- Reading digital and analog sensors - DHT11, BME280, soil moisture, IR/ultrasonic

- Push-button input handling and debouncing

## Hands-on Lab

- Wire and read DHT11, BME280, soil moisture, and ultrasonic sensors

- Display live sensor readings on the OLED via I2C

- Build a button-triggered LED control circuit

## Learning Outcomes

- Explain the differences between I2C, SPI, and UART, and choose the right one for a given peripheral

- Independently wire and read from each sensor type provided

- Display real-time data on an OLED screen

- Debug wiring and firmware issues using Serial Monitor output

# Day 2 - Concurrency, Power and Connectivity

## Topics

- FreeRTOS fundamentals - tasks, scheduling, priorities; why ESP32 firmware needs an RTOS

- Low-power modes - light sleep, deep sleep, wake sources, battery-life tradeoffs

- WiFi on ESP32 - connecting, handling disconnects/reconnects

- Building a web server on ESP32 - handling HTTP GET/POST requests

- Introduction to MQTT - publish/subscribe model, broker setup, topics

- MQTT vs. HTTP - when lightweight pub/sub beats request/response, and vice versa

## Hands-on Lab

- Refactor Day 1's sensor-reading sketch into separate FreeRTOS tasks (e.g. sensor read + display update running concurrently)

- Implement deep sleep with timer-based wake; observe and compare power behaviour

- Stand up a web server on the ESP32 exposing sensor data and GPIO control endpoints

- Connect the ESP32 to a local MQTT broker; publish sensor readings and subscribe to a control topic

- Parse and display live GPS coordinates on the OLED

## Learning Outcomes

- Structure ESP32 firmware using FreeRTOS tasks instead of a single blocking loop

- Apply appropriate low-power modes for battery-driven scenarios

- Expose device data and controls over HTTP directly from the ESP32

- Publish and subscribe to MQTT topics, and explain when to prefer MQTT over HTTP

- Retrieve and parse live location data from a GPS module

# Day 3 - Capstone 1: Hospital Helpdesk Voice Chatbot

## Topics

- Voice pipeline architecture - microphone capture (I2S/INMP441), speech-to-text, AI reasoning, text-to-speech, speaker output (I2S/MAX98357)

- REST API fundamentals - requests, query parameters, JSON responses

- Connecting ESP32 audio capture to an n8n workflow over HTTP (audio payload upload) and MQTT (lightweight status/event signaling)

- Database schema design for a hospital helpdesk: doctors, beds, wards, OPD/emergency status, patients

- AI Agent tool design - mapping spoken intents to database queries

## Hands-on Lab / Capstone Build

- Set up a database (spreadsheet-backed or lightweight SQL) with tables for: doctors (name, specialty, fee, availability), beds (ward, number, occupied/free), patients (name, bed number, ward), and OPD/emergency queue status

- Build an n8n workflow: ESP32 mic - speech-to-text - AI Agent - database lookup tools - text-to-speech - ESP32 speaker

- Implement AI tools for: doctor lookup and fees, OPD/emergency status, locating a specific ward, and finding a patient's bed number

- Use MQTT to signal device status (e.g. listening, processing) to the OLED in real time during HTTP audio upload

## Learning Outcomes

- Build a complete voice-in, voice-out pipeline on ESP32-class hardware

- Design a relational data model for a real institutional use case

- Combine HTTP (audio transfer) and MQTT (status signalling) appropriately within one system

- Translate natural spoken queries into structured database lookups via AI tool-calling

# Day 4 - Capstone 2: Library AI Voice Assistant

## Topics

- Database integration patterns for inventory systems - books, copies, status (available/issued/reserved)

- Business rules as AI Agent instructions - loan limits, due dates, reservation priority

- Lightweight identification - spoken or typed user name/ID, no physical credential hardware

- Book allocation logic - matching a request to an available copy, updating inventory state

- OLED menu design - building a simple on-device navigable menu with push buttons

- IR-based low-power activation - using the IR/ultrasonic sensor as a presence trigger to wake the device from low-power mode

## Hands-on Lab / Capstone Build

- Build a book inventory database (title, author, copies available, copies issued, due dates)

- Implement borrowing rules in the AI Agent's instructions (e.g. maximum 3 books per user, 14-day loan period)

- Build the voice/text interaction flow: user states name and request - AI checks rules and inventory - allocates or declines - confirms back via speech

- Build an OLED menu (browse by category, check my borrowed books, return a book) navigated with push buttons

- Wire the IR/ultrasonic sensor to wake the ESP32 from deep sleep when a person approaches the kiosk, combining Day 2's low-power work with this capstone

## Learning Outcomes

- Encode real-world business rules as AI Agent instructions rather than hard-coded firmware logic

- Build and update a stateful inventory system through natural-language requests

- Design a usable on-device menu system with limited I/O (OLED + buttons)

- Combine presence sensing with low-power modes for an always-ready, battery-friendly kiosk device

# Day 5 - Capstone 3: AI-Assisted Home Power Management System

## Topics

- Monitoring and controlling home power distribution per room/unit

- Solar vs. grid source switching - control logic and relay/actuator integration

- WhatsApp as a conversational interface - Meta Developer account, WhatsApp Business Cloud API, two-way messaging in n8n

- Time-series data logging - storing sensor and usage logs with timestamps in a spreadsheet/database

- Billing estimation models - converting usage data into projected cost

- Basic load and usage prediction from historical log data

- Leakage analysis - flagging abnormal consumption patterns against expected baselines

## Hands-on Lab / Capstone Build

- ESP32 reports temperature, humidity (DHT11/BME280), and occupancy (IR/ultrasonic) per room to an n8n workflow on a scheduled interval

- Workflow pulls external weather data for context (e.g. solar generation likelihood based on cloud cover/sunlight)

- Implement solar/grid switching logic: AI Agent reasons over weather + time of day + load to decide source, and triggers a relay accordingly

- Every reading and decision is logged with a timestamp into a spreadsheet/database - building a real usage history during the session

- AI Agent estimates running cost from logged usage and flags any room/unit whose consumption deviates sharply from its own recent baseline (leakage indicator)

- Connect WhatsApp via the Meta Developer test number: the owner can ask questions or send commands by message, and receives proactive alerts unprompted

- GPIO 2 acts as the proof-of-concept actuator (pump/load) for the live demo

## Learning Outcomes

- Design a multi-room monitoring system with timestamped historical logging

- Implement automated source-switching logic driven by both sensor and external API data

- Build a two-way conversational interface over WhatsApp using the Meta Developer platform

- Translate raw usage logs into billing estimates and basic anomaly/leakage detection

- Integrate scheduled automation with on-demand conversational override in a single system

# Final Deliverables

By the end of the program, each participant or team will have built and demoed three independent, working systems:

- A voice-driven hospital helpdesk chatbot with live database lookups (Day 3)

- A voice/text library assistant with inventory management, rule enforcement, and a low-power on-device menu (Day 4)

- A multi-room home power management system with solar/grid switching, billing estimation, leakage detection, and two-way WhatsApp control (Day 5)

Each team retains their hardware kit, full source code, n8n workflow exports, and database/spreadsheet templates to extend independently after the workshop.

# Demo Format

- Each capstone is demoed live at the end of its respective day

- Demos run against the real hardware and live data gathered during that day's session - no pre-recorded or simulated runs

- A short Q&A and design-critique round follows each demo, where teams explain key tradeoffs they made

IoT and AI Workshop

Protocols  |  RTOS  |  Voice AI  |  Power Automation

Build the future of connected intelligent devices.